### Reset en Equipos de sonido Sony

Esta es una lista o recopilación de cómo hacer reset o cold reset en diferentes modelos de equipos de audio o de sonido de la marca Sony.

Reset sirve para limpiar o borrar todos los datos incluidos preajustes (presets) almacenados en la RAM por el propietario, y la razón principal de hacer reset es regresar a condiciones iniciales o de fábrica el aparato.

Para efectuar un reset el equipo tiene que estar encendido y después de haber presionado los tres botones en forma simultánea, por lo general el display mostrará el mensaje “COLD RESET” y el reset estará hecho.

Algo a tomar en cuenta en esta lista es que muchos buscan estos modelos como "MHC" y la verdad todos los modelos aquí incluidos están como "HCD" lo cual también es valido, si eres observador puedes notar que en la parte frontal de estos equipos viene con modelo MHC y en la parte trasera como HCD, así que si lo buscas de una forma exacta ten en cuenta este detalle.

A continuación dejo algunos modelos y la forma de como resetear:

| MODELO | PRESIONAR SIMULTANEAMENTE |
| --- | --- |
| HCD-BX3 | STOP + ENTER + POWER |
| HCD-BX5 | STOP + ENTER + POWER |
| HCD-DR4 | CLOCK SET + ENTER + POWER |
| HCD-DR5 | CLOCK SET + ENTER + POWER |
| HCD-DR6 | CLOCK SET + ENTER + POWER |
| HCD-DR440 | CLOCK SET + ENTER + POWER |
| HCD-DX3 | STOP + ENTER + POWER |
| HCD-DX5 | STOP + ENTER + POWER |
| HCD-DX5J | STOP + ENTER + POWER |
| HCD-EC55 | PLAY MODE/TUNING MODE + FUNCTION + POWER |
| HCD-EC68 | STOP + FUNCTION + POWER |
| HCD-EC69 | STOP + CD + POWER |
| HCD-EC77 | PLAY MODE/TUNING MODE + FUNCTION + POWER |
| HCD-EC78 | STOP + FUNCTION + POWER |
| HCD-EC79 | STOP + CD + POWER |
| HCD-EC99 | STOP + CD + POWER |
| HCD-EX6 | STOP + FUNCTION + POWER |
| HCD-EX6T | STOP + FUNCTION + POWER |
| HCD-EX8 | STOP + FUNCTION + POWER |
| HCD-EX8T | STOP + FUNCTION + POWER |
| HCD-EX9 | STOP + FUNCTION + POWER |
| HCD-EX9T | STOP + FUNCTION + POWER |
| HCD-F50 | FUNCTION + ENTER + CD PLUS 1 |
| HCD-F100 | FUNCTION + ENTER + CD PLUS 1 |
| HCD-FR1 | FUNCTION + ENTER + CD PLUS 1 |
| HCD-GN600 | STOP + AMP MENU + POWER |
| HCD-GN660 | STOP + ENTER + POWER |
| HCD-GN770 | STOP + ENTER + POWER |
| HCD-GN800 | STOP + AMP MENU + POWER |
| HCD-GN880 | STOP + ENTER + POWER |
| HCD-GNX60 | STOP + ENTER + POWER |
| HCD-GNX70 | STOP + ENTER + POWER |
| HCD-GNX88 | STOP + ENTER + POWER |
| HCD-GNX600 | STOP + ILLUMINATION/SUB WOOFER LEVEL + POWER |
| HCD-GNX700 | STOP + ENTER + POWER |
| HCD-GNX800 | STOP + ENTER + POWER |
| HCD-GPZ6 | POWER + DISPLAY + STOP (CD) |
| HCD-GPZ7 | POWER + DISPLAY + STOP (CD) |
| HCD-GRX50 | STOP + ENTER/NEXT + POWER |
| HCD-GT22 | STOP + PLAY/PAUSE + GROOVE |
| HCD-GT44 | STOP + PLAY/PAUSE + GROOVE |
| HCD-GT55 | STOP + PLAY/PAUSE + GROOVE |
| HCD-GT111 | STOP + ENTER + POWER |
| HCD-GT222 | STOP + ENTER + POWER |
| HCD-GT444 | STOP + ENTER + POWER |
| HCD-GT555 | STOP + ENTER + POWER |
| HCD-GTR33 | STOP + ENTER + POWER |
| HCD-GTR55 | STOP + ENTER + POWER |
| HCD-GTR77 | STOP + ENTER + POWER |
| HCD-GTR88 | STOP + ENTER + POWER |
| HCD-GTX66 | STOP + ENTER + POWER |
| HCD-GTX77 | STOP + ENTER + POWER |
| HCD-GTX777 | STOP + DELAY + POWER |
| HCD-GTX787 | STOP + DELAY + POWER |
| HCD-GTX888 | STOP + DELAY + POWER |
| HCD-GX35 | STOP + GROOVE + POWER |
| HCD-GX40 | STOP + GROOVE + POWER |
| HCD-GX99 | PLAY MODE/TUNING MODE + FUNCTION + POWER |
| HCD-GX250 | STOP + PLAY MODE/TUNING MODE + DISC 1 |
| HCD-GX355 | STOP + ILLUMINATION + DISC 1 |
| HCD-GX450 | STOP + PLAY MODE/TUNING MODE + DISC 1 |
| HCD-GX470 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-GX555 | STOP + ILLUMINATION + DISC 1 |
| HCD-GX570 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-GX750 | STOP + PLAY MODE/TUNING MODE + DISC 1 |
| HCD-GX9000 | STOP + ENTER + POWER |
| HCD-GX9900 | STOP + ENTER + POWER |
| HCD-R770 | STOP + ENTER/NEXT + POWER |
| HCD-RG55 | STOP + GROOVE + POWER |
| HCD-RG55S | STOP + GROOVE + POWER |
| HCD-RG66T | STOP + ENTER + POWER |
| HCD-RG77 | STOP + ENTER + POWER |
| HCD-RG88 | STOP + ENTER + POWER |
| HCD-RG190 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-RG270 | STOP + ILLUMINATION+ DISC 1 |
| HCD-RG290 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-RG310 | STOP + GROOVE + POWER |
| HCD-RG330 | STOP + GROOVE + POWER |
| HCD-RG475 | STOP + ILLUMINATION+ DISC 1 |
| HCD-RG490 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-RG551 | STOP + PLAY MODE/TUNING MODE + DISC 1 |
| HCD-RG575 | STOP + ILLUMINATION + DISC 1 |
| HCD-RG590 | STOP + PLAY/PAUSE + DISC 1 |
| HCD-RX550 | STOP + PLAY MODE/TUNING MODE + DISC 1 |
| HCD-RXD7 | STOP + ENTER/NEXT + POWER |
| HCD-W300 | CLOCK SET + ENTER + POWER |
| HCD-W5000 | CLOCK SET + ENTER + POWER |
| HCD-XB6K | SPECTRUM ANALIZER + ENTER + DISC 1 |
| HCD-XB66 | SPECTRUM ANALIZER + ENTER + DISC 1 |
| HCD-XB500 | CLOCK SET + ENTER + POWER |
| HCD-XGR6 | CLOCK SET + TUNER ENTER + POWER |
| HCD-XGR60 | CLOCK SET + TUNER ENTER + POWER |
| HCD-XGR80 | CLOCK SET + TUNER ENTER + POWER |
| HCD-ZX6 | STOP(TAPE B) + TUNER ENTER + POWER |
| HCD-ZX8 | STOP(TAPE B) + TUNER ENTER + POWER |
| HCD-ZX66i | ►►(iPod) + TUNING MODE + POWER |
| HCD-ZX99i | ►►(iPod) + TUNING MODE + POWER |